Herbert Hoover Dike Culvert Replacement

S-272 (C-13)

InRoads DTM Files

Inroads Surfaces:

Reach1_ver8.dtm (existing condition)

C-13 Top of Muck.dtm	Existing Herbert Hoover Dike top of muck survey.

C-13 Bottom of Muck.dtm	Existing Herbert Hoover Dike hard bottom

C-13 Phase 1 Lake Side Cofferdam.dtm

Surface used to create earth cofferdam.  Top crest El. of earthen cofferdam is 24’ with a

lakeside slope of 1:4 and inside (landward) slope of 1:3 down to elevation 15’. At El. 15’ there is

a 15’ wide bench and a slope of 1:4 to existing grade.

C-13 Phase 1 Landside Cofferdam North Plug.dtm

Design of earthen cofferdam plug. This is built on existing grade (“C-13 Reach1_ver8.dtm”) land

side to elev. 21.5’, top width of 10’ with 1:4 inside slope (towards excavation pit) and 1:3

outside slope to existing grade.

C-13 Phase 1 Landside Cofferdam South Plug.dtm

Design of earthen cofferdam plug.  This is built on existing grade (“C-13 Reach1_ver8.dtm”) land

side to elevation 21.5’, top width of 10’ with 1:4 inside slope (towards excavation pit) and 1:3

outside slope to existing grade.

C-13 Phase 2 Excavation Pit.dtm

The surface design excavates the existing Herbert Hoover embankment surface to varying

elevations and to elevation -3’ using 1:4 side slopes.

C-13 Ph 4 Embnkmnt Rstore.dtm

Designed to restore the excavated Herbert Hoover embankment after the construction of new

S-272 structure.  The surface includes the construction of 2-14’ wide access ramps (12’

Shellrock, 2’ sod) with a minimum slope of 1:10 and 2- 12’ wide groin access ramps (12’ shell

rock) as well as working platforms around both structure head walls. Surface includes the repair

of adjacent pump station parking area and drainage.

C-13 Phase 4 Land side channel.dtm	Land Side approach channel

C-13 Ph 4 Gr.dtm	C-13 Phase IV Groin design.

Design slope.dtm

Design surface of slope, of top of bank elevation 19’ to toe elevation 4’. Area excavated to

perimeter of lake side cofferdam and transitioning to the surrounding existing conditions. The

Phase IV groin is designed to this design slope surface.

Inroads Alignments:

HHD_06-159.alg	Horizontal alignment of Herbert Hoover Dike.

C-13 Geomerty.alg	Centerline of the S-272 structure