Herbert Hoover Dike Culvert Replacement

S-271 (C-10A), InRoads Files

Accurate as of 03/04/15

ProjectWise location:	\Documents\C&SF\Herbert Hoover Dike Rehabilitation\03-Culvert Rehabilitation\S-271 (C10A)\070 - Plans\05 Civil\InRoads\

Project S-271.rwk :	This file is used to load surfaces (DTM’s) used, Alignments used, XIM file.

Surfaces Survey:

Location:	Documents\Survey data\C&SF\Herbert Hoover Dike\Reach 1\C-10A

C-10a_with_borings.dtm:	Existing surface including hard bottom, with boring information used to locate excavated areas on both sides of the outfall channel.

Surfaces:

By Pass Cofferdam 070214.dtm:	Designed lakeside earthen cofferdam for temporary bypass structure, transitioning from elev. 35.5’ at the existing levee to elev. 25.0’, top width of 12’ with 1:4 lakeside slopes and 1:4 landside slopes.

By Pass Excavation 022815.dtm:	This surface design excavates the existing Herbert Hoover embankment surface to elevation 0.0’ using 1:4 side slopes.  This will allow the installation of 3 10’ dia. Hdpe pipes, walkway and gates.

By Pass Canal Phase II 030415.dtm:	This surface design excavates the existing south toe ditch to elevation 4.0’ between driven sheet pile walls. This will allow the redirection of the L-8 canal through the By Pass Structure.

By Pass Reconst. 022715.dtm:	This surface design restores Herbert Hoover embankment surface to elevation 35.5’ using 1:4 side slopes, this surface also degrades the center portion of the By Pass Cofferdam to elevation 4.0’ for the construction of the By Pass outfall channel.

Transition Cofferdam 071714.dtm:	Designed lakeside earthen cofferdam with the southern portion connecting to the northern remaining portion of the By Pass cofferdam. This cofferdam has a bench at elev. 15.0’ and a top elevation of 25.0’, top width of 12’ with 1V:4H lakeside slopes and 1V:3H landside slope to the bench. The top of the cofferdam transitions from elev. 25.0’ to elev. 35.5’ at the existing levee. Also includes earthen plug with a top elevation of 21.0’ with 1V:4H slopes at the existing northern toe ditch.

Excavation Pit 061014.dtm:	This surface excavates the existing Herbert Hoover embankment to a elevation of -1.5’ beneath the existing structure, then sloping to a elevation of 6.0’ out to the end of the existing groins using side slopes of 1V:4H.

Partial Embankment Rest.dtm:	Designed to restore the excavated Herbert Hoover embankment after the installation of the permanent S-271 structure to elev. 35.5’ with a crest width of 16’, this restoration includes portions of the southern 14’ wide access ramps, construction of 14’ wide northern access ramps, a portion of the landside platform, construction of the lakeside platform, lakeside groins and the lakeside outfall.

Lake Side Rest. 092314dtm:	Designed to restore the lakeside outfall.

Land Side Outfall:	Designed to restore the landside outfall.

North Toe Ditch.dtm:	Reconstructed northern toe ditch, top of bank elev. 16.0’, bottome elev. 12.0’.

Phase IX Plug.dtm:	This surface design restores the center portion of the By Pass cofferdam.  This will allow the removal the 3 10’ dia. Hdpe pipes, walkway and gates.

Final Embankment Rest.dtm:	Designed to restore the excavated Herbert Hoover embankment after the removal the 3 10’ dia. Hdpe pipes, walkway and gates to elev. 35.5’ with a crest width of 16’, this restoration includes construction of the remaining portions of the southern 14’ wide access ramps and landside platform.

South Toe Ditch.dtm:	Reconstructed northern toe ditch, top of bank elev. 14.0’, bottom elev. 10.0’.

Quantity Surfaces:

The following surfaces are used to calculate earthwork quantities:

Phase II Completed.dtm	Existing surface, By Pass Cofferdam and By Pass Excavation merged.

Phase VII Completed.dtm	Existing surface, By Pass Cofferdam and By Pass Excavation merged.

Alignment:

C-10A Geometry.alg	Horizontal alignments of cofferdams and the reconstructed Herbert Hoover Dike.

HHD_06-159.alg:	Horizontal alignment of Herbert Hoover Dike.

XIM:

Civil Preferencel.xin:			This loads InRoad levels, symbology, and preferences